ULTRASOUND

Hazard location

Ultrasound is the mechanical vibration of an elastic medium that is produced in the form of alternating compressions and expansions. The vibration may be produced by continuous or impulse sound in the form of a sequel of interrupted vibrations. The medical uses of ultrasound include therapeutic surgical, and diagnostic procedures.

Potential health effects

Although exposure to ultrasound does not appear to pose a human health risk, exposure to audible high-frequency radiation above 10 kHz can result in a syndrome involving nausea, headaches, tinnitus, pain, dizziness, and fatigue. Temporary hearing loss and threshold shifts are also possible from high-frequency ultrasound radiation.

Low-frequency ultrasound radiation may produce local effects when a person touches parts of materials being processed by ultrasound. The hands are often involved in the area where ultrasound acts most strongly. Exposure to powerful sources of ultrasound may result in damage to peripheral nervous and vascular structures at the points of contact. Airborne ultrasound vibration may produce effects on the central nervous system and on other systems and organs through the ear and through extra-auditory routes.

Standards and recommendations

No OSHA standard or NIOSH recommendation exists for ultrasound. ACGIH has proposed the following TLVs for permissible exposure to airborne upper sonic and ultrasonic acoustic radiation (ACGIH 1987):

| Mid-frequency of third-octave band kHz | One-third octave-band level in dB re 20 microPa |
| --- | --- |
| 10 | 80 |
| 12.5 | 80 |
| 16 | 80 |
| 20 | 105 |
| 25 | 110 |
| 31.5 | 115 |
| 40 | 115 |
| 50 | 115 |

Exposure control methods

Exposure to ultrasonic vibration can be reduced by the use of enclosures and shields. Sound-isolating panels on ultrasonic equipment should be free of any openings and should be isolated from the floor by rubber seals. Workers operating or repairing ultrasonic equipment should be provided with appropriate protective equipment that is selected based on the task being performed and the likelihood of exposure to radiation above 10 kHz or to contact with low-frequency sources.

Video Display Terminals

Hazard location

Video display terminals, VDTs have rapidly replaced other word processing and data management systems in many hospital departments.

Potential health effects

VDT’s are a frequent source of worker complaints. Eyestrain, back, neck, and arm discomfort, and symptoms of stress have all been associated with VDT work. These problems may be controlled or improved with ergonomic measures such as adjusting the position of the screen and keyboard, the chair, the lighting and glare, the color contrast, and the frequency of rest periods. Whether long-term VDT use causes significant visual dysfunction or degeneration is unknown. Extensive radiation measurements and health data have indicated that VDT's do not appear to present a radiation hazard to the operators (Pomroy and Noel 1984) or to the developing fetuses of pregnant operations (NIOSH 1984a). However, clusters of miscarriages and birth defects have been reported among VDT operators and warrant further investigation (NIOSH 1984a).

Recommendations

NIOSH studies have resulted in a report entitled Potential Health Effects of Video Display Terminals (NIOSH 1981h) which contains specific recommendations for the installation, maintenance, and use of VDT's. NIOSH recommends the following general guidelines for VDT work (NIOSH 1984a).

Workstation design: VDT units, supporting tables, and operator chairs should be designed with maximum flexibility. VDT's should have detachable keyboards, and work tables should be adjustable for height. Chairs should be adjustable for height and should provide proper back support.

- Illumination: Sources of glare should be controlled through VDT placement, i.e. parallel to windows, and parallel to and between lights, proper lighting, and the use of glare-control devices on the VDT screen surface. For VDT tasks requiring screen-intensive work, illumination levels should be lower than those needed when working with hard copy, which may require local lighting in addition to normal office lighting.

- Work regimens: Continuous work with VDT's should be interrupted periodically by rest breaks or other work activities that do not produce visual fatigue or muscular tension. As a minimum, a break should be taken after 2 hr of continuous VDT work. Breaks should be more frequent as visual, mental, and muscular burdens increase.

- Vision testing: VDT workers should have visual testing before beginning VDT work and periodically thereafter to ensure that they have adequately corrected vision to handle such work.